5/5/10

COMPLAY 4.0.1

RELEASE NOTICE

# General

All components of Complay have been compiled for this release which includes the Main

Complay Application, all Controls, and all Plug-Ins.

It is necessary to Uninstall any existing versions of Complay4.0, Complay3.5 or

Complay3.0 prior to Installing this version. See Note 1.

# Enhancements and Fixes

MAIN APPLICATION

Enhancements

- Create a new Display Type for Vanquish-HD displays.

- Support for running under Windows7. Note that performance is not up to par with

that of running under Windows-XP however there is no impact on functionality. The

performance will be improved in a future release.

## Fixes

- None

TRANSMISSION CONTROL

Enhancements

- Direct TCP/IP network communications is supported for the Vanquish-HD. Network

communication for the HX1 and Z180 cpu types must still go through Lantronix.

- Shows the status of the downloaded bytes in Kbytes instead of Mbytes.

Fixes

- AVI’s greater than 32MB would fail during transmission. This has been corrected.

FRAME EDITOR

Enhancements

- None

Fixes

- None

PLUG-INS

Enhancements

- The Reset and Clear/Download Firmware Plugin will now allow downloading over a

Network connection.

Fixes

- None

INSTALLATION

Enhancements

- Keying in the word “DEMO” as the W/O# into the Serial# field (“485453-DEMO”) will

cause the Unlock Code logic to execute after 30 days just as if they had keyed in a

real W/O#. After 45 days the user will have had to call HiTech for an Unlock Code or

the software will become locked out and unusable. This is meant to be used for

potential customers who want a copy of Complay4 for evaluation. Note that the clock

does not start until Complay4 actually communicates with one of our displays.

- JMS Privatized Version Install has been created. Everything is functionally equivalent

with HiTech’s standard version.

Fixes

- Problems related to updating the Registry incorrectly during Install were corrected.

These were possibly causing other issues now and then when trying to Uninstall

Complay and then Install a later version of Complay.

DEPENDENCIES

- Current Vanquish-HD Firmware Emulator Software is V2.21

- Current HX1 Firmware is V1.45B

- Current Z180/Z2 Firmware is V2.18B

WARNINGS:

- Always Uninstall previously Installed versions of Complay4.0, Complay3.5, and

Complay3.0 before Installing this version.

- You can download the Vanquish-D application to an HX1 display and the HX1

firmware to a Vanquish-HD display. If this is done the display will become inoperable

and will require a trip to the site to correct. Be very careful that you do not do this.

KNOWN ISSUES/PROBLEMS

- Windows7 performance issues.

- Display File Manager Send File function won’t allow uploading of avi’s > 32 MB.

- It is possible to download the Vanquish-D application to an HX1 display and the HX1

firmware to a Vanquish-HD display. This results in an inoperable display.

- In GetInfo the Frame_Size and Tot_Frames is treated as a signed # and should

be unsigned and the Last Variable Update does not display correctly.

- Setting a Delay of >= 500 seconds does not delay the correct amount of time.

- In Preview, if the Hour is < 10 (IE – 1 – 9) a leading 0 is displayed (IE - 08:32).

- Rename Prefix/Suffix “Scroll” Left, Right, Up, Down to “Shift” Left, Right, Up,

Down.

- Priority in the On Board Scheduler is not functional and will be removed in a future

release.

- Doing Properties/Version on Complay4.exe shows version 4.0.0.1 and it should be

version 4.0.1.0

NOTES:

Note 1

- Complay4.0 will install itself into the Complay3 directory therefore it is still

necessary to Uninstall Complay3.5 or Complay3.0 before Installing another

version of Complay4.0, Complay3.5, or Complay3.0.

While progress has been made so that you can Install a new version over an old

version the best solution for now is always to Uninstall Complay4.0, Complay3.5, or

Complay3.0 before Installing a new version. If problems exist after Uninstalling

Complay4.0, Complay3.5, or Complay3.0 and Installing the new version of Complay4.0

follow the procedure below.

- Uninstall Complay4.0, Complay3.5, or Complay3.0 using Control Panel, Add Remove Programs.  If either appears there twice,  Uninstall it again.

- Navigate to “Program files\Hitech\Complay 3\”

- Delete “Complay_x.exe” from “Program files\Hitech\Complay 3\” directory if it is there.

- Delete “C3_Display_Scheduler.exe” and “C3_Scheduler_Monitor.exe” from “Program files\Hitech\Complay 3\Scheduler\”.

- Delete all .dll, .ocx, .oca from “Program files\Hitech\Complay 3\bin\”

- Delete all .dll, .ocx, .oca from “Program files\Hitech\Complay 3\plugins\”

- Delete any references to “Program files\Hitech\Complay 3\” (shared dlls) with REGEDIT. See Note 2.

- Install Complay4.0

Note 2

- Using REGEDIT to clean up the Shared DLL’s Registry. This should be done

only with the assistance of qualified HiTech personel.

When a component (.dll.  ocx,  .exe) is marked as shared the Windows Installer will

create an entry for it in

“HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows\CurrentVersion\

SharedDlls\”

Each time a program that uses that DLL is installed the reference count will increase

by 1.  Each time a program is uninstalled the reference count will be decreased by 1.

Normally this is not a problem unless an incomplete install or uninstall occurs.  This

can cause the uninstall to not completely remove the old product.  When the new

product is installed it will not overwrite the old component and a software mismatch will

occur causing all kinds of strange problems. To clean up the Shared DLL directory

perform the following.

- From the Command Prompt or “Start-Run” execute “REGEDIT”

- Navigate to “HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows\CurrentVersion\

SharedDlls\”

- Highlight and Delete any references to “Program Files\HiTech\Complay 3\”

- Exit REGEDIT.